Please complete the design and verification for the following “SIMPLE SWITCH” block

in1

out0

out2

out1

in0

SIMPLE SWITCH

in1

out3

Specification:

- in0, in1 – inputs, 8-bit wide. out0-3 – outputs, 8-bit wide

- All ports work on common clock named – “clk”

- The “rst” port which is synchronous to “clk”, resets all logic within the Simple Switch

- The input data consists of 3-bytes forming one packet as shown below, can come continuously, in three clock cycles, with one clock cycle gap between two packets coming to the same input port

Byte 2

Byte 1

Byte 0

Byte 0 – Header, Bytes 1,2 – Payload

The bits 7,6 of Byte-0 decides on which output port the packet (Bytes 0 – 2) should be directed

E.g. 00 – the packet should be directed to output port 0 (out0), 01 – output port 1 (out1)….11 – output port 3

- The packets can come on any of the input ports (in0 or in1) and even arrive at the same time in both the ports

Design and Verification:

- Design the “SIMPLE SWITCH” to perform as per the specification mentioned above

  - Think about all different conditions that can occur in the SWITCH

- Draw the block diagram, FSM state transition diagram and timing diagram in your report

- Provide explanations where needed

- Write the RTL for your design

- Verify the design with either directed Verilog or SV testbench using random data generation

- Summarize your design and verification experience